Project Proposal

On

CLePS: A Cross-site, Leakage-safe and Persistence-Skill Evaluation Framework for Machine-Learning PV Power Forecasting

MASTER OF TECHNOLOGY

IN

Artificial Intelligence and Machine Learning

SUBMITTED BY

Niranjan Dhanyakumar Kolpe

(PRN - 25070149016)

Guide: Dr. Shivali Wagle. Co-guide: Dr. Madhvi Saxena.

SYMBIOSIS INSTITUTE OF TECHNOLOGY, PUNE – 412115

(A CONSTITUENT OF SYMBIOSIS INTERNATIONAL (DEEMED UNIVERSITY))

2026-27

# INTRODUCTION

## Background

A photovoltaic (PV) plant converts sunlight into electricity. Its output, PV power (in kW), depends largely on weather, mainly the solar radiation reaching the panels. It therefore rises and falls with the sun's daily path and with passing clouds, and it is zero at night.

PV power forecasting predicts this output for a future time. The time between issuing a forecast and the time it refers to is the forecast horizon, which ranges from minutes (intra-hour) to a day or more (day-ahead) [1], [21]. Forecasts help grid operators and plant owners balance supply and demand, schedule energy storage and trade electricity [2].

Machine-learning (ML) models, such as gradient-boosted trees and neural networks (for example the multilayer perceptron, MLP, and the long short-term memory network, LSTM), learn the relation between inputs (past PV power, measured or forecast weather, time of day) and future PV power from historical data. Deep learning dominates recent research; for example, 58 of the 119 studies reviewed in [2] use deep-learning models.

Key terms used in this proposal:

- Training, validation and test data: a model is fitted on training data, its settings (hyperparameters) are tuned on validation data, and its accuracy is reported on test data that it has never seen. The test result is meant to show how the model will perform in real use.

- Error metrics: RMSE (root mean square error) and MAE (mean absolute error) measure the average size of forecast errors in kW. Normalised RMSE (nRMSE) divides RMSE by the rated capacity of the plant, so that plants of different size can be compared.

- Persistence forecast: the simplest forecast, which assumes the future value equals the present one, for example power in one hour equals power now [43]. Smart persistence instead keeps the clear-sky index constant, that is, the ratio of measured power to the power expected under a cloudless sky at that time, so the forecast follows the sun's daily path [42], [43]. It is the usual reference that any useful model must beat. Example (illustrative values): at 10:00 a plant gives 60 kW while its clear-sky power is 80 kW (clear-sky index 0.75); if clear-sky power at 11:00 is 96 kW, persistence forecasts 60 kW for 11:00 and smart persistence forecasts 0.75 x 96 = 72 kW.

- Forecast skill: FS = 1 - RMSE(model) / RMSE(reference forecast), usually with smart persistence as the reference [42], [43]. FS above 0 means the model is better than the reference (FS = 0.2 means 20% lower RMSE); FS below 0 means it is worse. Unlike RMSE alone, skill can be compared across plants and periods with different weather. Example (illustrative values): if smart persistence has an RMSE of 50 kW and the model 40 kW, FS = 1 - 40/50 = 0.2.

- Data leakage: a spurious relationship between the model inputs and the target that comes from how the data were collected, sampled or pre-processed, so that test accuracy looks better than it would be in real use [46]. Recognised forms include pre-processing (for example scaling) fitted on training and test data together, inputs that would not legitimately be available when the prediction is made, and temporal leakage, where test data are not strictly later in time than training data [46]. In PV forecasting, random splits of a time series are flagged as a leakage risk [10], because neighbouring time steps, which are nearly identical, end up in both training and test data. Example (illustrative): if a year of 5-min data is shuffled and split at random, a test row at 12:05 on 3 June may have training rows at 12:00 and 12:10 on either side, so the model nearly copies the answer.

- Leakage-safe split: a split divides the data into training, validation and test sets; it is leakage-safe when no information from the test set reaches model building. For time series this means three rules: the sets follow time order (training earliest, test latest); every pre-processing step, such as scaling, is fitted on training data only; and model settings are chosen on validation data only, never on test data. Example (illustrative values): with one year of data, January to August is training, September is validation and October to December is test; the scaling mean (say 42 kW) is computed from January to August only and applied unchanged to the later months.

- Target-adjacent input: a measured variable that is almost a direct component of the target. Inverter current is an example, since AC power is roughly voltage times current. Measured at the same instant as the target, such an input nearly gives the answer away; this is a form of illegitimate input [46]. Example (illustrative values): with an AC voltage of 230 V and an output current of 20 A measured at 12:00 on a single-phase inverter, power at 12:00 is about 230 x 20 = 4.6 kW, so the "forecast" needs no model.

- Cross-site testing: testing a model at a plant (site) that was not used for training. It shows whether a model works at a new plant, for example a newly built plant with little or no history [40], [43]. Example: a model trained on data from two plants is tested on a third plant whose data it never saw during training.

## Motivation

Because reported test accuracy is the main way ML models are compared, weaknesses in how it is measured make published results hard to trust or compare. A PRISMA 2020 review of 39 reviews (Scopus, 2021-2026), described in Section 2, reports 17 recurring research gaps. Three of them concern how accuracy is measured and are addressed in this project:

- Single-site testing: 63% of studies use one site [2]; most of 100 ML studies were tested on one system in one mild climate [23].

- Data leakage in evaluation: only 37% of 119 PV forecasting studies address temporal leakage prevention [2]; random k-fold splits are flagged as a leakage risk [10].

- Weak baselines: naive persistence is described as too weak a reference, and stronger reference methods are called for [14]; evaluation by forecast skill is recommended but uncommon [21].

Each weakness has a direct consequence. Testing on a single site says nothing about performance at a new plant. Leakage makes accuracy look better than it would be in real use. Without a persistence reference, a low error may come from easy, sunny weather rather than from the model, and results from different plants cannot be compared. Example (illustrative values): at a sunny plant a model with an RMSE of 20 kW can still be worse than smart persistence (RMSE 15 kW, skill -0.33), while at a cloudy plant a model with an RMSE of 40 kW can beat it (RMSE 50 kW, skill 0.2); RMSE alone would rank them the wrong way round.

A targeted search for primary studies found none that addresses all three weaknesses together (Section 2.4). This project therefore proposes CLePS (Cross-site, Leakage-safe and Persistence-Skill), an evaluation framework for ML PV power forecasting that addresses all three in one protocol.

# LITERATURE REVIEW

## Systematic Literature Review/Bibliometric Review

The review followed PRISMA 2020 and used Scopus only. Search A combined PV power terms, forecasting terms (forecast*, predict*, estimat*, nowcast*) and ML terms, limited to 2021-2026, articles, conference papers and reviews, English, final publication stage and open access, which gave 4,067 records. The 235 reviews among them were screened by title and then by abstract (records marked Include or Unsure went forward), 57 were sought in full text, 53 were read in full and 39 were included.

Search B checked novelty. It added three blocks of terms to the same query: leakage-safe splitting, multiple sites, and persistence or skill-score baselines. Requiring all three blocks returned 0 records. Requiring at least two returned 33 records; after the same filters (articles and conference papers only), screening and full-text checks, 6 primary studies were included. Figure 1 shows both flows.

Figure 1. PRISMA 2020 flow diagram for Search A (reviews) and Search B (primary studies)

## Review of methods available in the literature

The 39 included reviews [1]-[39] are compared in Table 1 (focus, method, dataset and research gap of each; full details are in the accompanying Literature Review document). Together they cover deep learning, for example transformers [6] and LSTM networks [27], hybrid physics-ML models [16], explainable AI [5] and uncertainty modelling [4].

Table 1. Comparison of the 39 included reviews (Search A)

| Study (Author, Year, Citation) | Focus | Method | Dataset | Research Gap |
| --- | --- | --- | --- | --- |
| Mahaini et al., 2026 [1] | ML PV/wind, ≥24 h | SLR, Scopus, 31 studies | SCADA (measured); DKASC, SolarDB (public) | Consistent benchmarking; forecast-weather (NWP) inputs; uncertainty reporting; decision-oriented evaluation; compute cost; interpretability |
| Wedhasari and Castro, 2026 [2] | Validation, leakage, reproducibility | Structured review, 119 studies | SunLAB, DKASC, ESOLMET (on-site) | Leakage prevention (37%); little cross-validation; seeds unfixed; fragmented metrics, few skill scores; single-site (63%); economic value |
| Jadapalli et al., 2026 [3] | AI irradiance and PV forecasting | Scoping review, 60 studies | None named | Cross-climate/cross-site benchmarking; uncertainty; transformer data needs; open data, standard protocols; retraining, explainability, robustness |
| Cui et al., 2026 [4] | PV uncertainty modelling | Taxonomy review | ERA5 (30-yr hourly reanalysis) | Multi-timescale consistency; data fusion; physics-constrained generation; extreme scenarios; cross-regional generalization; unified benchmark |
| Tufail, 2026 [5] | Explainable AI | SLR, 4 databases | NSRDB, DKASC, Stanford sky images | Quantitative XAI evaluation; fragile benchmarking; code/data sharing (1.7%); multi-site validation; human-in-the-loop testing; causal models |
| Sharma et al., 2026 [6] | Transformer variants | Review + 3-model benchmark | Vadodara 10 MW plant (daily, 2015-17) | No public benchmark; uncertainty (conformal); physics-informed attention; NWP fusion; multi-site graph models; missing/noisy data |
| Matrenin et al., 2026 [7] | Trustworthy forecasting architecture | SLR (PRISMA 2020) | None named | Semantic data models; end-to-end architecture; shallow XAI; drift governance; lifecycle automation; clean-data evaluation; non-standard metrics |
| Sadeq et al., 2026 [8] | BIPV potential, power, energy | SLR (PRISMA), 6 databases | None named | Unified PPE framework; urban generalization; inconsistent metrics; short horizons; techno-economics; façade validation data |
| Singh et al., 2026 [9] | ML, DL and hybrid methods | Narrative review | DTU, Kaggle AMS, Oklahoma Mesonet | Single-site, small data; multi-site long-term data; regional model; cloudy days; DL-physical hybrids; normalised metrics |
| Tabatabaei and Antonini, 2025 [10] | Urban rooftop/BIPV ML | Review + meta-analysis (k=4) | OSM, PVGIS, ERA5 (recommended) | Equatorial/arid data; spatial and external-city validation (k-fold leakage); domain adaptation; uncertainty; benchmarks; compute; economic/carbon KPIs |
| Di Leo et al., 2025 [11] | Model classification and selection | Narrative review | SKIPP'D (1-min sky images, 3 yr) | Multi-climate benchmarks; standard metrics incl. skill; cross-validation, significance tests; data fusion; adaptive learning; compute; interpretability; data quality |
| Khouili et al., 2025 [12] | DL architectures and features | SLR, WoS, 26 studies | None named | Cross-region generalization; limited data; model compression; grid integration; explainability; probabilistic forecasting; standard indicators |
| Husein et al., 2024 [13] | DL PV forecasting, 2022-2023 | SLR, WoS + Scopus, 36 studies | NREL, NSRDB, DKASC; mostly private | Public benchmarks; scale-independent metrics; cross-location models; shared PV database; aerosol/cloud inputs |
| Barhmi et al., 2024 [14] | Irradiance and PV techniques | Narrative review, Google Scholar | NSRDB, SURFRAD, BSRN, SKIPP'D | Benchmark with baselines beyond naive persistence; public datasets; climate-based site classification; XAI; extreme weather |
| Al-Dahidi et al., 2024 [15] | Data-driven PV framework | Framework-based review | GEFCom2014 | Integrated pipelines; concept drift; incremental learning; prediction intervals; application metrics; real case studies |
| Santos et al., 2024 [16] | Hybrid physics-ML | SLR, IEEE + Scopus, 67 studies | None named (single plants) | Hybrid benchmark; unequal inputs in comparisons; night hours lower errors; few probabilistic, long-term hybrids; horizon-wise analysis |
| Zhou et al., 2024 [17] | Methods and trends | Narrative review | DKASC Alice Springs | Extreme/cloudy weather; season-specific models; slow optimisation, weak clustering; distributed PV with topography |
| Yu et al., 2024 [18] | DL architectures | Narrative review, 55 models | NREL Solar-1/2/3, PVDAQ, DKASC (public) | No climate-general architecture; GNN complexity; weak multivariate generalization; input selection |
| Sankari and Kumar, 2024 [19] | Smart-city solar forecasting | Narrative survey | None | Weather/sky-imager inputs; feature selection; transfer learning; hyperparameter tuning |
| Gallardo et al., 2023 [20] | Real-time PV prediction | SLR, WoS, 60 studies | None named | Standard testbench (data, horizons, metrics); more data, feature selection; embedded real-time hardware |
| Pandžić and Capuder, 2024 [21] | Short-term ML, open-data benchmark | Citation-based review | NREL SRRL, SKIPP'D, SURFRAD (open) | Closed data; non-comparable results; longer tests, common metrics and baselines; probabilistic forecasting; explainability |
| Jannah et al., 2024 [22] | Methods and public PV datasets | Keyword-search review | PVOD, SOLETE, NIST, UNISOLAR (7 public) | Cross-site generalization; inconsistent metrics; no benchmarks; data quality; weather sensitivity; tuning cost; embedded deployment |
| Alcañiz et al., 2023 [23] | Trends and gaps in ML PV forecasting | Classification review, 100 papers | None named (mostly single systems) | Unseen sites and climates; few tropical/dry studies; high-resolution data; standard metrics; interpretability, robustness, post-processing |
| Deng et al., 2023 [24] | PV scenarios, methods, investment | Descriptive survey | Meteosat, AVHRR, FY-2C (satellite) | Cloud growth and motion; costly image labelling; hand-crafted features; NWP inaccuracy in cloudy weather |
| Divya and Umamaheswari, 2022 [25] | Direct and indirect forecasting | Descriptive survey | None named | None stated by authors |
| Cesar et al., 2022 [26] | Spatio-temporal GHI/PV | Keyword-search review | Ground pyranometer/PV networks (mostly USA) | Public multi-site quality data; open sharing; stronger baselines; probabilistic, longer-horizon, hybrid work |
| Jailani et al., 2023 [27] | LSTM and LSTM hybrids | Narrative comparison | DKASC Alice Springs, Texas 34 sites | Differing metrics, weather, horizons; code/data rarely shared; medium/long-term horizons |
| Iheanetu, 2022 [28] | Data-driven PV techniques | Narrative survey | None named | Weather-forecast dependence; compute for longer horizons; input selection; degradation losses; standard metrics |
| Mohamad Radzi et al., 2023 [29] | Neural network models | Narrative review | Alice Springs, Yulara sites | Longer-horizon accuracy; weather classification; input selection; overfitting; small data; RNN gradients |
| Gaboitaolelwe et al., 2023 [30] | Short-term ML vs persistence | Review + experiment | Utrecht rooftop PV (175 systems, 1-min), ECMWF | Normalised errors and site reporting; location transferability; adverse weather; NWP inputs; deep and probabilistic models |
| Dimd et al., 2022 [31] | Nordic cold-climate PV | Narrative review | NTNU Trondheim rooftop PV | Snow soiling and albedo; snow data; cross-horizon studies; regional upscaling |
| Wu et al., 2022 [32] | ML/DL approaches and optimisers | Narrative review | None named | NWP integration; attention models rare; data-collection mix; noisy/missing data; probabilistic forecasting |
| Liu et al., 2022 [33] | Temporal and spatial scales | Narrative taxonomy | None named | No uniform evaluation standard; few regional studies; nonlinear models; feature extraction; regional upscaling |
| Gupta and Singh, 2021 [34] | Direct vs indirect data-driven | Narrative review | None named | No universal model; DL underused; accuracy vs compute; wavelet choice; new plants without history |
| Alkabbani et al., 2021 [35] | Metaheuristic-tuned ML | Categorization review | None named | Regional multi-site; probabilistic, multistep; undisclosed input-target links; longer horizons; metaheuristic training time |
| Massaoudi et al., 2021 [36] | DL taxonomy, probabilistic | SLR (PRISMA) | DKASC, NREL, NIST | Cross-site generalization; standard metrics; real-world validation; data fusion; online learning; interpretability; longer horizons |
| Saltos et al., 2026 [37] | Short-term hybrid AI | Database-search review | None named | Inconsistent metrics, little uncertainty; open data, benchmarks; cross-regional validation; edge models; XAI; SCADA/EMS integration |
| Pedram et al., 2025 [38] | Building PV (BIPV/BAPV) | Narrative review, Scopus, 89 articles | NSRDB, PVGIS, PVOutput, DKASC | Standard benchmarks; normalised metrics; imputation; spatio-temporal models; online/federated learning; feature selection; EMS evaluation; BIPV data |
| Rodriguez-Aburto et al., 2025 [39] | ML PV with energy storage | SLR + meta-analysis (k=5) | None named | Standard benchmarks; external validation; uncertainty; domain-shift, missing-data robustness; ESS control loop; XAI; synthetic data |

The 39 reviews fall into five groups by focus. Each group is described below, ending with the gaps its reviews state.

### Broad reviews of ML and DL for PV forecasting

Twelve broad reviews map ML and DL methods for PV forecasting: medium-term PV and wind forecasting [1], validation and leakage practice across 119 studies [2], AI for irradiance and PV [3], ML, DL and hybrid methods [9], model classification [11], DL studies from 2022-2023 [13], methods and trends [17], 100 ML studies classified by method and climate [23], direct and indirect forecasting [25], data-driven techniques [28], ML and DL approaches with optimisers [32] and data-driven models [34]. Their most frequent gaps are single-site testing, with most models trained and tested on one plant in one mild climate [3], [9], [23], and the lack of a standard benchmark or common metrics [2], [13], [32]. They also report weak leakage prevention (only 37% of studies address it) and few skill scores against persistence [2], scarce uncertainty quantification [1], [3], untested statistical significance [11] and a need for methods for new plants without history [34]; one review states no explicit gaps [25].

### Reviews of specific model families

Nine reviews focus on specific model families: transformers [6], deep-learning architectures [12], [18], [36], hybrid physics-ML models [16], LSTM models [27], neural networks [29], metaheuristic-tuned ML [35] and hybrid AI models [37]. Their most recurrent gaps are poor generalization across regions and sites [12], [16], [36], [37] and the lack of a public benchmark or standard metrics [6], [37]. They also note that comparisons often give models unequal inputs and that night hours lower reported errors [16], that no single architecture suits all climates [18], that code and data are rarely shared [27], that input selection is unclear [29], [35] and that lightweight edge models are rare [37].

### Reviews of uncertainty, explainability and trust

Four reviews address uncertainty, explainability and trust: uncertainty modelling [4], explainable AI [5], trust features in forecasting systems [7] and a data-driven framework with incremental learning [15]. Their recurrent gaps are concept drift and adaptation over time [4], [7], [15] and unreliable explanations, with explainable-AI outputs rarely evaluated quantitatively [5], [7]; they also call for prediction intervals [15] and report that only 1.7% of studies share both code and data [5].

### Reviews of data, benchmarks and real-time systems

Five reviews focus on data, benchmarks and real-time systems: forecasting techniques and benchmarks [14], real-time prediction systems [20], short-term ML with an open-data benchmark [21], public PV datasets [22], and a review that tests ML methods against persistence on one Dutch dataset [30]. Their recurrent gaps are weak baselines, with naive persistence too weak a reference [14] and common baselines and metrics missing [21], and poor cross-site generalization [22], [30]; they also call for a standard testbench and embedded on-site hardware [20] and note that most data are closed [21].

### Reviews of specific settings and applications

Nine reviews address specific settings: building-integrated PV [8], [38], urban rooftop PV [10], smart cities [19], grid-connected PV systems [24], spatio-temporal forecasting [26], Nordic cold climates [31], multiple time and space scales [33] and PV with energy storage [39]. Their recurrent gaps are the lack of standard benchmarks and normalised metrics [33], [38], [39] and under-studied extreme or climate-specific conditions [10], [31]. They also call for spatial and external-city validation instead of random k-fold splits, which risk leakage [10], stronger baselines and public multi-site data [26], and external validation [39].

## Summary of Literature Review

Potential research gaps reported in the 39 reviews

- CROSS-SITE GENERALIZATION: Most models are trained and tested on one plant in one climate [3, 9, 12, 16, 22, 23, 30, 36, 37]; solving it shows whether a model built elsewhere works on a new plant, including plants with no history.

- DATA LEAKAGE IN EVALUATION: Few studies report safeguards against leakage: only 37% address temporal leakage prevention [2], and random k-fold splits are flagged as a leakage risk [10]; related evaluation flaws, such as unequal model inputs and night-time zeros, also inflate reported accuracy [16]. Solving it gives a true measure of accuracy.

- NO STANDARD BENCHMARK OR METRICS: Studies use different datasets, test periods and error measures, so results cannot be compared [2, 6, 13, 32, 33, 37, 38, 39]; solving it gives a fair way to rank models.

- WEAK OR MISSING BASELINES: Baselines are often missing or too weak: naive persistence is a weak reference [14], and common or stronger baselines are called for [21, 26]; solving it shows whether ML adds real value over a simple reference forecast.

- MEASURED VS FORECAST WEATHER INPUTS: Most studies use measured weather as input instead of weather forecasts available at prediction time [1]; solving it shows the accuracy achievable in real operation.

- OPEN DATA AND REPRODUCIBILITY: Few studies share data or code (e.g. 1.7% share both [5]; 8% use public data [2]; also [21, 26]); solving it lets others verify and build on results.

- UNCERTAINTY QUANTIFICATION: Most forecasts give a single value without a confidence range [1, 3, 15, 37, 39]; solving it lets grid operators plan reserves and manage risk.

- FORECAST VALUE FOR DECISIONS: Accuracy is rarely linked to cost, dispatch or storage decisions (~98% report no operational value [2]; also [1, 39]); solving it shows what better forecasts are worth in money and grid stability.

- CONCEPT DRIFT AND ADAPTIVE LEARNING: Models are rarely tested as seasons, panel ageing or operating conditions change [4, 7, 15]; solving it keeps forecasts reliable over a plant's lifetime.

- ROBUSTNESS TO REAL-WORLD DATA DEFECTS: Models are trained on clean data; missing or faulty measurements are rarely tested [7, 11, 32, 38]; solving it makes models usable on real plant data streams.

- EXTREME AND CLIMATE-SPECIFIC CONDITIONS: Extreme weather, snow and non-temperate climates are under-studied [4, 10, 17, 23, 31]; solving it gives accurate forecasts when the grid needs them most.

- INPUT SELECTION: Which inputs genuinely improve forecasts is unclear and inputs are often chosen by trial and error [23, 28, 35]; solving it gives simpler models with inputs justified by evidence.

- REGIONAL AND DISTRIBUTED PV: Few studies forecast many small or regional PV systems together [9, 17, 35]; solving it helps grid operators who manage thousands of rooftop systems.

- STATISTICAL SIGNIFICANCE OF GAINS: Reported improvements are rarely tested for statistical significance [11]; solving it shows whether a better-scoring model is truly better.

- EXPLANATION RELIABILITY: Explainable-AI outputs are rarely checked for faithfulness or stability [5, 7]; solving it lets engineers trust why a model predicts what it does.

- LONGER FORECAST HORIZONS: Most work is short-term; multi-day and seasonal forecasts are scarce [1, 26, 27]; solving it supports market bidding and maintenance planning.

- LIGHTWEIGHT AND EDGE DEPLOYMENT: Few models are tested on real-time hardware at the plant [20, 37]; solving it makes on-site forecasting cheap and practical.

## Chosen research gaps

- CROSS-SITE GENERALIZATION: 63% of studies use a single site [2], and most of 100 ML studies were tested on one system in one mild climate [23].

- DATA LEAKAGE IN EVALUATION: Only 37% of 119 PV forecasting studies address temporal leakage prevention [2], and random k-fold splits are flagged as a leakage risk [10].

- WEAK OR MISSING BASELINES: Naive persistence is described as too weak a reference, and stronger reference methods are called for [14]; evaluation against forecast skill is recommended but uncommon [21].

Gaps 1, 2 and 4 of the list above were chosen because they are preconditions for valid evaluation. The other gaps, such as uncertainty, explainability, concept drift, edge deployment and longer horizons, are judged through reported test accuracy, which leakage, single-site testing and a missing persistence reference can each make misleading. Gap 5 (measured versus forecast weather inputs) is related to leakage, since weather measured at the target time is not available when a forecast is issued; CLePS avoids it by using only values known at the issue time. The three gaps are also linked, because a fair test at a new site needs both a leakage-safe split and a persistence reference. Each is reported by at least three of the 39 reviews, cross-site generalization by the most (9), and Search B found no study that addresses all three.

Table 2 compares the six closest primary studies found by Search B [40]-[45].

Table 2. Comparison of the six closest primary studies (Search B) and their coverage of the three gaps

| Study | Focus | Method | Dataset | Research Gap | Leakage-safe split | Multiple sites | Persistence / skill baseline |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Xu et al., 2025 [40] | New-site 5-min forecasting | Sky-image Swin Transformer, transfer learning | Stanford SKIPP'D to UQ Gatton (1-min) | Satellite and weather inputs | No: hand-picked test days, random train/validation split | Partial: target site fine-tuned | Yes |
| Perera et al., 2024 [41] | Day-ahead regional rooftop PV | Hierarchical temporal CNNs | Solar Analytics, Western Australia (hourly, private) | Transformer and linear models; compute cost; retraining for new systems; BIPV | Yes: chronological hold-out | Partial: 101 postcodes, all used in training | Yes |
| Paletta et al., 2024 [42] | Zero- and few-shot new-site transfer | Sky-image ConvLSTM | SIRTA Paris to SKIPP'D Stanford (1-min, public) | Clear-sky model accuracy; representative calibration days; more cloud/aerosol types and locations | No: fixed test days, ten-fold cross-validation | Yes: unseen target site | Yes |
| Sarmas et al., 2022 [43] | New-plant 1-h forecasting | Stacked LSTM, transfer learning | Portuguese energy community (hourly) | Source data volume; multiple source plants; geographic differences | Partial: scaling before split; source training period overlaps target test period | Yes: 1 source, 6 target plants | Yes |
| Ilias et al., 2023 [44] | Plants without production data | LSTM-transformer, domain adaptation | Portuguese energy community (hourly) | Semi-supervised adaptation; accuracy variation across plants; weather changes and faults | Partial: validation and scaling not reported | Yes: 1 source, 6 target plants | Yes |
| Luo et al., 2022 [45] | Day-ahead forecasting with 30 days of data | Constrained LSTM, transfer learning | GEFCom2014, Australia (hourly, ECMWF inputs) | Large source dataset needed; same input features required; other regions | Partial: source model selected on test error | Partial: 2 plants | Yes |

None of the six covers all three gaps. [41] uses a chronological split and a persistence baseline but never tests on an unseen site. [42] tests on an unseen site against smart persistence but uses fixed test days and ten-fold cross-validation. [40] uses hand-picked test days with a random train/validation split and fine-tunes on the target site. [43], [44] and [45] cover several plants and persistence but have leakage risks or unreported steps: scaling before the split and overlapping source and target periods in [43], validation and scaling not reported in [44], and a source model selected on its test error in [45]. The gap addressed by this project is therefore a leakage-safe, cross-site evaluation referenced to persistence.

# PROBLEM STATEMENT

### Problem statement

In short-term PV power forecasting, the reported accuracy of machine-learning models is often inflated and hard to compare across studies, because evaluations commonly allow data leakage, test only on plants seen in training and omit skill against a persistence reference, and how much each of these practices changes the reported skill has not been measured.

Type of problem: supervised time-series regression; the target is PV power at time t + h (h = 15 min and 1 h) and the inputs are values known at time t.

Techniques: smart persistence (reference), ridge regression, gradient-boosted trees, multilayer perceptron (MLP) and long short-term memory network (LSTM) (Section 4.3).

Datasets: four candidate PV plants with timestamps, AC power and inverter current at 5-min resolution or finer, in four locations (Section 4.4, Table 5).

Limitations: horizons are short-term (15 min and 1 h), because archived weather forecasts are not available for the candidate plants; the final set of plants depends on the data sufficiency check (Section 4.2); the novelty claim is limited to the literature covered by the two Scopus searches.

### Objectives

- Objective 1: To develop CLePS (Cross-site, Leakage-safe and Persistence-Skill), an evaluation framework for ML PV power forecasting that combines a leakage-safe split, testing on plants not used for training, and forecast skill against smart persistence.

- Objective 2: To quantify, per plant and per model, how much reported forecast skill is overstated by:

  - Data leakage through random splitting, full-data scaling, test-set tuning or inverter current measured at the target time.

  - Testing only on plants used for training, measured as skill lost on unseen plants.

- Objective 3: To identify which models still beat smart persistence under leakage-safe, cross-site evaluation.

### Contributions

- Contribution 1 (CLePS framework): one evaluation protocol that combines a leakage-safe split, cross-site testing and forecast skill against smart persistence; none of the six closest studies covers all three (Table 2).

- Contribution 2 (Leakage inflation): measured skill inflation from four leakage paths (random split, full-data scaling, test-set tuning, inverter current at the target time), per plant and per model.

- Contribution 3 (Cross-site loss): measured skill lost from within-site to leave-one-plant-out testing, per plant and per model.

# METHODOLOGY

## General methodology

The method has six steps (Figure 2). Sections 4.2 to 4.4 give the details and the reason for each choice.

- Data: raw data of PV plants that meet the requirements of Section 4.4 are loaded.

- Standardized dataset format: a small converter per dataset writes each plant to one standard CSV file (Table 3); a sufficiency check keeps only plants with enough valid data; CLePS then builds the samples from these files (Section 4.2).

- Leakage-safe split: chronological training, validation and test blocks per plant; every fitted step uses training data only, and settings are chosen on validation data only.

- Models: smart persistence (reference), ridge regression, gradient-boosted trees, MLP and LSTM (Table 4).

- Experiments: Experiment 1 (E1), leakage test; Experiment 2 (E2), cross-site test; Experiment 3 (E3), skill against smart persistence.

- Metrics and outputs: RMSE weights large errors, which matter most for grid balancing; MAE gives the typical error; nRMSE (divided by plant capacity) lets plants of different size be compared [13], [38]; forecast skill against smart persistence removes the effect of easy, sunny weather and is the comparison recommended in [21] and used in [42], [43]. Every run is saved as one row of a CSV results file.

# Figure 2. Flowchart of the proposed method; the dashed box marks the CLePS evaluation framework

## Detailed methodology

Pre-processing and standard format: a converter for each dataset applies four steps:

- Converts timestamps to UTC, because plants lie in different time zones and daylight saving shifts local time.

- Averages data finer than 5 min to 5-min means, keeping an interval only if at least 80% of its raw points exist; data coarser than 5 min are rejected, because the 15-min horizon and 5-min inputs need them.

- Sets physically impossible values (negative power, power above rated capacity, constant "stuck" readings) to empty.

- Sums the phase currents.

Plant details (capacity, latitude, longitude and source; tilt and azimuth if known) are kept in one file, plants.csv. Normalised power, sun position and clear-sky power are computed by CLePS (pvlib) and not stored, so any dataset with the three columns of Table 3 can enter CLePS.

Table 3. Standardized dataset format (one CSV file per plant)

| Column | Data type | Unit | Content |
| --- | --- | --- | --- |
| timestamp_utc | text, ISO 8601, UTC | - | Start of the 5-min interval |
| power_kw | decimal (float) | kW | AC power, 5-min mean (target) |
| current_a | decimal (float) | A | Inverter output current, summed over phases, same device as power_kw (used only in leakage path (d)) |

Inputs, target and samples: for issue time t, the inputs are the 12 normalised power values from t - 55 min to t and sun position and time of day at t and t + h, which are known in advance; the target is normalised power at t + h. Weather is not used, because E2 needs the same inputs at every plant and weather sensors differ between datasets. A sample is used only if the sun is at least 5° above the horizon at t and t + h, because night-time zeros lower the reported errors [16] and smart persistence is undefined at night; the filter is applied to samples, not to the series, so night values never become neighbours of day values. Input gaps of up to three steps are filled from the last valid value (past data only); targets are never filled, because a filled target is not real ground truth. All models and smart persistence are scored on the same samples.

Data sufficiency: raw record counts cannot be compared across resolutions (100 records at 1 s cover 100 s), so sufficiency is counted in valid samples after processing. A plant is used only if its data span at least 6 months and each of the training, validation and test blocks has at least 20 valid days (a day is valid if at least 70% of its daytime samples are valid); 6 months give about 20,000 samples. A sufficiency report (CSV) lists each plant as used or excluded. The 80%, 70% and 20-day thresholds are design choices.

Split: each plant is split in time order: first 60% training, next 20% validation, last 20% test (a common ratio). Standardization is fitted on training data only; hyperparameters are chosen on validation RMSE; the chosen model is scored once on the test block.

Experiment 1 (E1), leakage test: each leakage path is switched on alone and compared with the leakage-safe run:

  - Random split: rows are shuffled before a random 60/20/20 split.

  - Full-data scaling: scaling is fitted on all data, including test data.

  - Test-set tuning: hyperparameters are chosen on test data.

  - Target-adjacent input: inverter current at t + h is added as an input [46].

Skill inflation = skill with the path minus skill of the leakage-safe run. For path (a) the test set itself changes, so the inflation is the difference in reported skill, as a reader of a paper would see it.

Experiment 2 (E2), cross-site test: for each plant, the within-site model is compared with a model trained on the training and validation blocks of the other plants (leave-one-plant-out) and tested on the same test block; scaling is fitted on the pooled training data of the other plants. Cross-site loss = skill (within-site) minus skill (leave-one-plant-out). Plants must lie at separate sites, so that they do not share weather.

Experiment 3 (E3) and outputs: every result is reported as forecast skill = 1 - RMSE(model) / RMSE(smart persistence), with RMSE, MAE and nRMSE. Models with random elements (gradient-boosted trees, MLP, LSTM, and the random split in (a)) run with five seeds (a common compromise between run time and stable estimates); results are mean and standard deviation. Each run writes one row (plant, horizon, model, setting, leakage path, seed, number of test samples, RMSE, MAE, nRMSE, skill) to the results CSV, from which all tables and figures are produced.

## Method/Model Description

The models range from a physical reference to linear, tree-ensemble, feed-forward and recurrent models, so that the effect of leakage and of cross-site testing can be seen across model complexity (Table 4). All ML models get the same inputs, a fairness issue raised in [16]. Transformers are left out, because their data needs are a concern [3] and the aim is to test evaluation, not to find the best model. Each ML model gets a small hyperparameter grid of similar size, searched on validation data only.

Table 4. Models and reasons for their choice

| Model | What it is | Why it is included |
| --- | --- | --- |
| Smart persistence | Keeps the clear-sky index at t constant up to t + h [42], [43] | Reference for forecast skill; recommended in [21], used in [42], [43]; needs no training |
| Ridge regression | Linear model with an L2 penalty | Fast, transparent linear baseline; shows the skill reachable without non-linearity; regularised linear regression is benchmarked in [30] |
| Gradient-boosted trees | Ensemble of shallow trees fitted in sequence | Captures non-linear effects with little pre-processing; among the most used ML models for PV forecasting [1], [9], [30] |
| MLP | Feed-forward neural network | Basic neural model for PV forecasting [18], [29], [30] |
| LSTM | Recurrent neural network with memory of earlier steps | Leading deep-learning family for PV forecasting [13], [27]; deep learning dominates recent work (58 of 119 studies in [2]) |

## Datasets Description

Requirements: a dataset is usable if it has:

- Timestamps with a known time zone (chronological split, sun position).

- AC power (the target).

- Inverter current at the same device and time as the power (leakage path (d)).

- Capacity and location (normalisation, clear-sky power for smart persistence); tilt and azimuth are used if known, otherwise a horizontal clear-sky curve is used.

- A resolution of 5 min or finer.

- At least 6 months of data (Section 4.2).

Cross-site testing (E2) needs at least three plants at separate sites. Four candidate plants meet requirements 1, 2, 3, 5 and 6 (Table 5) and their locations are known; capacities are taken from the documentation of each dataset.

Table 5. Candidate datasets

| Plant | Location | Period | Resolution | Current |
| --- | --- | --- | --- | --- |
| NIST Ground array [47] | Maryland, USA | 2017 | 1 min | DC and 3 AC phases |
| HKUST, LSK North [48] | Hong Kong | May 2021 - Dec 2023 | 5 min | 3 AC phases |
| La Réunion plant (about 2 kW, single-phase) [49] | La Réunion, France | Oct 2021 - Oct 2022 | 5 min | Grid (AC) current |
| PVDAQ system 9068, inverter 1 [50] | Kersey, Colorado, USA | Jan 2024 - Apr 2025 | 5 min | AC phases per module |

Tools: Python with pandas, scikit-learn (ridge, MLP), LightGBM (gradient-boosted trees), PyTorch (LSTM) and pvlib (sun position, clear-sky power).

# RESEARCH PLAN

| Task No. | Task Description | Duration | Status |
| --- | --- | --- | --- |
| 1 | Domain identification and topic exploration | Jul - Oct 2026 | Completed |
| 2 | PRISMA Search A: 235 reviews screened, 39 included | Oct 2026 | Completed |
| 3 | Search B novelty check: 6 primary studies included | Oct 2026 | Completed |
| 4 | Topic proposal (this document) | Oct 2026 | Submitted for approval |
| 5 | Implementation: Data converters, standard files and sufficiency check; E1 leakage experiments (ridge regression, gradient-boosted trees, 1 h horizon) | 20-21 Oct 2026 | Planned |
| 6 | E2 cross-site experiments | 21-22 Oct 2026 | Planned |
| 7 | Initial draft of the research paper | By 22 Oct 2026 | Planned |
| 8 | Progress report | 25 Oct 2026 | Planned |
| 9 | ESE presentation | 30 Oct 2026 | Planned |
| 10 | Full experiments (MLP, LSTM, 15 min horizon) and final paper | After 30 Oct 2026 | Planned |

# REFERENCES

- D. Mahaini, M. R. Machado, and J. L. R. Moreira, “Machine learning for wind and solar PV energy production forecasting: a systematic literature review,” International Journal of Data Science and Analytics, vol. 22, no. 1, 2026, doi: 10.1007/s41060-026-01261-z.

- T. Wedhasari and R. Castro, “A Critical Review and Strategic Roadmap of PV Power Forecasting (2016–2026): Addressing Temporal Leakage and Operational Integration Gaps,” Energies, vol. 19, no. 12, 2026, doi: 10.3390/en19122937.

- S. Jadapalli et al., “Scoping review of artificial intelligence approaches for solar energy forecasting addressing methodological developments data practices and operational implications,” Discover Applied Sciences, vol. 8, no. 6, 2026, doi: 10.1007/s42452-026-08691-1.

- Y. Cui, X. Lv, C. Zhang, X. Zhao, and X. Fu, “AI-Based Multi-Timescale Photovoltaic Power Scenario Generation and Forecasting: A Statistical Relational Perspective,” Applied Sciences (Switzerland), vol. 16, no. 14, 2026, doi: 10.3390/app16147202.

- S. Tufail, “Explainable AI in solar PV forecasting: A systematic review of models, explainers, and physical drivers,” Solar Compass, vol. 19, 2026, doi: 10.1016/j.solcom.2026.100182.

- D. Sharma, C. Kumar, M. Kumar, and B. B. Singla, “A review of deep learning transformer variants used in solar PV forecasting,” Sustainable Energy Research, vol. 13, no. 1, 2026, doi: 10.1186/s40807-026-00270-7.

- P. V. Matrenin, I. F. Iumanova, and A. I. Khalyasmaa, “Conceptual Architecture of a Trustworthy Wind and Photovoltaic Power Forecasting System: A Systematic Review and Design,” Inventions, vol. 11, no. 1, 2026, doi: 10.3390/inventions11010015.

- M. Sadeq, F. Muhammad-Sukki, and N. Sellami, “Forecasting the Potential, Power and Energy (PPE) of Both Building Integrated PV and Traditional PV (BIPV–PV) Systems Using State-of-the-Art AI Methods,” International Journal of Energy Research, vol. 2026, no. 1, 2026, doi: 10.1155/er/1140262.

- U. Singh, N. Singh, H. Malik, and M. A. Hossaini, “Photovoltaic Power Forecasting: A Review on Models and Future Research Directions,” IET Renewable Power Generation, vol. 20, no. 1, 2026, doi: 10.1049/rpg2.70186.

- M. Tabatabaei and E. Antonini, “Machine Learning for Optimizing Urban Photovoltaics: A Review of Static and Dynamic Factors,” Sustainability (Switzerland), vol. 17, no. 18, 2025, doi: 10.3390/su17188308.

- P. Di Leo, A. Ciocia, G. Malgaroli, and F. Spertino, “Advancements and Challenges in Photovoltaic Power Forecasting: A Comprehensive Review,” Energies, vol. 18, no. 8, 2025, doi: 10.3390/en18082108.

- O. Khouili, M. Hanine, M. Louzazni, M. A. L. Flores, E. G. Villena, and I. Ashraf, “Evaluating the impact of deep learning approaches on solar and photovoltaic power forecasting: A systematic review,” Energy Strategy Reviews, vol. 59, 2025, doi: 10.1016/j.esr.2025.101735.

- M. Husein, E. Gago, B. Hasan, and M. Pegalajar, “Towards energy efficiency: A comprehensive review of deep learning-based photovoltaic power forecasting strategies,” Heliyon, vol. 10, no. 13, 2024, doi: 10.1016/j.heliyon.2024.e33419.

- K. Barhmi, C. Heynen, S. Golroodbari, and W. van Sark, “A Review of Solar Forecasting Techniques and the Role of Artificial Intelligence,” Solar, vol. 4, no. 1, pp. 99 - 135, 2024, doi: 10.3390/solar4010005.

- S. Al-Dahidi et al., “Forecasting Solar Photovoltaic Power Production: A Comprehensive Review and Innovative Data-Driven Modeling Framework,” Energies, vol. 17, no. 16, 2024, doi: 10.3390/en17164145.

- L. d. O. Santos, T. AlSkaif, G. C. Barroso, and P. C. M. d. Carvalho, “Photovoltaic power estimation and forecast models integrating physics and machine learning: A review on hybrid techniques,” Solar Energy, vol. 284, 2024, doi: 10.1016/j.solener.2024.113044.

- D. Zhou, Y. Liu, X. Wang, F. Wang, and Y. Jia, “Research Progress of Photovoltaic Power Prediction Technology Based on Artificial Intelligence Methods,” Energy Engineering: Journal of the Association of Energy Engineering, vol. 121, no. 12, pp. 3573 - 3616, 2024, doi: 10.32604/ee.2024.055853.

- J. Yu et al., “Deep Learning Models for PV Power Forecasting: Review,” Energies, vol. 17, no. 16, 2024, doi: 10.3390/en17163973.

- S. S. Sankari and P. S. Kumar, “Solar Power Forecasting in Smart Cities using Deep Learning Approaches: A Review,” International Research Journal of Multidisciplinary Technovation, vol. 6, no. 6, pp. 145 - 158, 2024, doi: 10.54392/irjmt24610.

- I. Gallardo, D. Amor, and Á. Gutiérrez, “Recent Trends in Real-Time Photovoltaic Prediction Systems,” Energies, vol. 16, no. 15, 2023, doi: 10.3390/en16155693.

- F. Pandžić and T. Capuder, “Advances in Short-Term Solar Forecasting: A Review and Benchmark of Machine Learning Methods and Relevant Data Sources,” Energies, vol. 17, no. 1, 2024, doi: 10.3390/en17010097.

- N. Jannah, T. S. Gunawan, S. H. Yusoff, S. A. Hanifah, and S. N. Mohd Sapihie, “Recent Advances and Future Challenges of Solar Power Generation Forecasting,” IEEE Access, vol. 12, pp. 168904 - 168924, 2024, doi: 10.1109/ACCESS.2024.3496120.

- A. Alcañiz, D. Grzebyk, H. Ziar, and O. Isabella, “Trends and gaps in photovoltaic power forecasting with machine learning,” Energy Reports, vol. 9, pp. 447 - 471, 2023, doi: 10.1016/j.egyr.2022.11.208.

- X. Deng, F. Da, H. Shao, and X. Wang, “A Survey of the Researches on Grid-Connected Solar Power Generation Systems and Power Forecasting Methods Based on Ground-Based Cloud Atlas,” Energy Engineering: Journal of the Association of Energy Engineering, vol. 120, no. 2, pp. 385 - 408, 2023, doi: 10.32604/ee.2023.023480.

- R. Divya and S. Umamaheswari, “Solar Power Forecasting Methods – A Review,” International Journal of Advanced Science and Engineering, vol. 9, no. 1, pp. 2591 - 2598, 2022, doi: 10.29294/IJASE.9.1.2022.2591-2598.

- L. B. Cesar, R. A. E. Silva, M. Á. M. Callejo, and C. Cira, “Review on Spatio-Temporal Solar Forecasting Methods Driven by In Situ Measurements or Their Combination with Satellite and Numerical Weather Prediction (NWP) Estimates,” Energies, vol. 15, no. 12, 2022, doi: 10.3390/en15124341.

- N. L. M. Jailani et al., “Investigating the Power of LSTM-Based Models in Solar Energy Forecasting,” Processes, vol. 11, no. 5, 2023, doi: 10.3390/pr11051382.

- K. J. Iheanetu, “Solar Photovoltaic Power Forecasting: A Review,” Sustainability (Switzerland), vol. 14, no. 24, 2022, doi: 10.3390/su142417005.

- P. N. L. Mohamad Radzi, M. N. Akhter, S. Mekhilef, and N. Mohamed Shah, “Review on the Application of Photovoltaic Forecasting Using Machine Learning for Very Short- to Long-Term Forecasting,” Sustainability (Switzerland), vol. 15, no. 4, 2023, doi: 10.3390/su15042942.

- J. Gaboitaolelwe, A. M. Zungeru, A. Yahya, C. K. Lebekwe, D. N. Vinod, and A. O. Salau, “Machine Learning Based Solar Photovoltaic Power Forecasting: A Review and Comparison,” IEEE Access, vol. 11, pp. 40820 - 40845, 2023, doi: 10.1109/ACCESS.2023.3270041.

- B. D. Dimd, S. Voller, U. Cali, and O. Midtgard, “A Review of Machine Learning-Based Photovoltaic Output Power Forecasting: Nordic Context,” IEEE Access, vol. 10, pp. 26404 - 26425, 2022, doi: 10.1109/ACCESS.2022.3156942.

- Y. Wu, C. Huang, Q. Phan, and Y. Li, “Completed Review of Various Solar Power Forecasting Techniques Considering Different Viewpoints,” Energies, vol. 15, no. 9, 2022, doi: 10.3390/en15093320.

- C. Liu, M. Li, Y. Yu, Z. Wu, H. Gong, and F. Cheng, “A Review of Multitemporal and Multispatial Scales Photovoltaic Forecasting Methods,” IEEE Access, vol. 10, pp. 35073 - 35093, 2022, doi: 10.1109/ACCESS.2022.3162206.

- P. Gupta and R. Singh, “PV power forecasting based on data-driven models: a review,” International Journal of Sustainable Engineering, vol. 14, no. 6, pp. 1733 - 1755, 2021, doi: 10.1080/19397038.2021.1986590.

- H. Alkabbani, A. Ahmadian, Q. Zhu, and A. Elkamel, “Machine Learning and Metaheuristic Methods for Renewable Power Forecasting: A Recent Review,” Frontiers in Chemical Engineering, vol. 3, 2021, doi: 10.3389/fceng.2021.665415.

- M. Massaoudi, I. Chihi, H. Abu-Rub, S. S. Refaat, and F. S. Oueslati, “Convergence of Photovoltaic Power Forecasting and Deep Learning: State-of-Art Review,” IEEE Access, vol. 9, pp. 136593 - 136615, 2021, doi: 10.1109/ACCESS.2021.3117004.

- J. M. Saltos, M. G. Intriago Cedeño, N. R. Balderramo Velez, G. T. R. León, and A. Cano-Ortega, “Hybrid AI Models for Short-Term Photovoltaic Forecasting: A Systematic Review of Architectures, Performance, and Deployment Challenges,” Sensors, vol. 26, no. 6, 2026, doi: 10.3390/s26061793.

- O. Pedram, A. Soares, and P. Moura, “A Review of Methodologies for Photovoltaic Energy Generation Forecasting in the Building Sector,” Energies, vol. 18, no. 18, 2025, doi: 10.3390/en18185007.

- C. Rodriguez-Aburto et al., “Machine Learning for Photovoltaic Power Forecasting Integrated with Energy Storage Systems: A Scientometric Analysis, Systematic Review, and Meta-Analysis,” Energies, vol. 18, no. 23, 2025, doi: 10.3390/en18236291.

- S. Xu, H. Ma, C. Ekanayake, and Y. Cui, “Swin transformer-based transferable PV forecasting for new PV sites with insufficient PV generation data,” Renewable Energy, vol. 246, 2025, doi: 10.1016/j.renene.2025.122824.

- M. Perera, J. De Hoog, K. Bandara, D. Senanayake, and S. Halgamuge, “Day-ahead regional solar power forecasting with hierarchical temporal convolutional neural networks using historical power generation and weather data,” Applied Energy, vol. 361, 2024, doi: 10.1016/j.apenergy.2024.122971.

- Q. Paletta, Y. Nie, Y.-M. Saint-Drenan, and B. Le Saux, “Improving cross-site generalisability of vision-based solar forecasting models with physics-informed transfer learning,” Energy Conversion and Management, vol. 309, 2024, doi: 10.1016/j.enconman.2024.118398.

- E. Sarmas, N. Dimitropoulos, V. Marinakis, Z. Mylona, and H. Doukas, “Transfer learning strategies for solar power forecasting under data scarcity,” Scientific Reports, vol. 12, 2022, doi: 10.1038/s41598-022-18516-x.

- L. Ilias, E. Sarmas, V. Marinakis, D. Askounis, and H. Doukas, “Unsupervised domain adaptation methods for photovoltaic power forecasting,” Applied Soft Computing, vol. 149, 2023, doi: 10.1016/j.asoc.2023.110979.

- X. Luo, D. Zhang, and X. Zhu, “Combining transfer learning and constrained long short-term memory for power generation forecasting of newly-constructed photovoltaic plants,” Renewable Energy, vol. 185, pp. 1062-1077, 2022, doi: 10.1016/j.renene.2021.12.104.

- S. Kapoor and A. Narayanan, “Leakage and the reproducibility crisis in machine-learning-based science,” Patterns, vol. 4, 2023, Art. no. 100804, doi: 10.1016/j.patter.2023.100804.

- M. Boyd, “High-Speed Monitoring of Multiple Grid-Connected Photovoltaic Array Configurations,” NIST Technical Note 1896, National Institute of Standards and Technology, Gaithersburg, MD, USA, 2015, doi: 10.6028/NIST.TN.1896.

- Z. Lin, Q. Zhou, Z. Wang, C. Wang, D. B. Bookhart, and M. Leung-Shea, “A high-resolution three-year dataset supporting rooftop photovoltaics (PV) generation analytics,” Dryad, 2024, doi: 10.5061/dryad.m37pvmd99.

- A. Graillet, C. Lebreton, C. Tang, F. Kbidi, T. Jegado, C. Damour, and M. Benne, “Electrical production data of a domestic grid-connected rooftop PV plant in normal and shading faults conditions associated with solar and meteorological data in a tropical climate,” Data in Brief, vol. 46, Art. no. 108723, 2023, doi: 10.1016/j.dib.2022.108723.

- C. Deline, K. Perry, M. Deceglie, M. Muller, W. Sekulic, and D. Jordan, “Photovoltaic Data Acquisition (PVDAQ) Public Datasets,” Open Energy Data Initiative (OEDI), National Renewable Energy Laboratory, 2021, doi: 10.25984/1846021.